PERSEUS SOFTWARE V1.0 RELEASE NOTES

© 2007, 2008 Microtelecom s.r.l. - Pavia di Udine, Italy

Date:		28 February 2008

Author:		Nico Palermo, IV3NWV

- 1 MSPS / 800 kHz BANDWIDTH PLAYBACK/RECORDING CAPABILITIES

The new sample rate option has been added to the Sampling Rate menu.

As the processing power is in some way proportional to the data sample rate this option requires

good CPUs to be run comfortably.

For this sample rate a Dual Core Pentium with at least 500 MB RAM and a 2.5 GHz clock is suggested.

- NOISE REDUCTION BLOCK ADDED

The noise reduction dsp block can be operated in all modes except with DRM and USER demodulators.

- USER DEMODULATOR

When the user demodulator is selected the VAC output is fed with zero-IF IQ samples, filtered with the selected

bandwidth and normalized in amplitude with the selected AGC setting.

This mode is useful when a third party application wishes to process the zero-IF data stream at the

selected frequency.

- SELECTIVITY FILTERS

The selectivity filter settings are now saved and remembered for each mode.

AM and Synchronous AM share the same filter.

USB and LSB shares the same filter.

The 6-dB bandwidth of the selectivity filter is now shown near the label BW in the upper-right corner

of the demodulation window.

The Variable Bandwidth (selectivity) Filter form factor has been largely improved.

For the SSB filter when a 2400 Hz 6dB bandwidth is selected, the 60dB stop bandwidth is 2600 Hz (F6/60=1:1.08)

- PASS BAND TUNE - NOTCH FUNTIONS

All the functions already available in the previous software versions are available

when the PBT button is selected in the secondary spectrum monitor window, accordingly to

the following table:

PBT MODE (PBT selected)

ACTION					EFFECT

Double click of mouse left btn			=> move selected point to 0 Hz offset

Drag with mouse left btn clicked			=> fine tune

Drag filter edges with mouse left btn clicked  	=> adjust selected selectivity filter edge

Drag with mouse right btn clicked               		=> shift filter mask left/right

Mouse wheel                                     		=> adjust selectivity filter bandwidth

Actions related to the notch functions can be activated in two ways, either performing the action

with the keyboard CRTL key pressed OR selecting the Notch Button (in this case the CTRL key is not

required) as listed in the following table:

NOTCH MODE (PBT selected and Ctrl-key down or NOTCH selected)

ACTION					EFFECT

Double click of mouse left btn			=> place notch at the selected frequency

Drag with mouse left btn clicked			=> tune notch filter to desired frequency

Right click                                     			=> disable notch filter

Mouse wheel                                     		=> adjust notch filter width

- WAV FILES PLAYBACK

The playback position can now be selected left clicking the record/playback bar in the appropriate point,

without dragging the mouse.

Playback loops are selected in the usual way, dragging the mouse (left button clicked) over the  recorder bar.

The Playback bar now display the start and stop UTC time of the file played.

- INCREASED PEAK AUDIO LEVEL TO VAC

VAC output peak level increased to -6 dBFs

- AUDIO BOARD SAMPLE RATE SYNCHRONIZATION

Audio output synchronized with input sample rate (only audio board, not VAC). In playback mode the playback rate is controlled by VAC if it present on the system (in this case the audio output is synchronized to VAC) or by the audio board itself if VAC is not present on the system.

- AUDIO OUTPUT LATENCY

Audio output latency set to approx. 200 ms for the 3 kHz filter (about 300 ms for the 0.8 kHz filter)

Latency can vary accordingly to the selected options. The synchronizer lock time has been kept relatively

slow and it may take several seconds for the loop to synchronize. During this lock time audio buffers

underruns/overflows may be experienced.

NOTE: Do not drive third party software with the audio output (i.e. selecting the default sound board

as the input source! Third party software should ALWAYS get its input from the Perseus VAC output.

- FREQUENCY SLIDE BAR

Decimal digits shown on the frequency slidebar labels when they are required

- INCREASED FFT SPECTRAL RESOLUTION

On the main display control bar the three buttons "Time", "Mix" and "Freq" allow to select

the "sharpness" of the FFT displays to three increasing values at the expense of the display refresh rate.

This function is particularly useful when analyzing strong carrier signals and carrier feed-through on adjacent

bins has to be reduced at minimum. For such analysis the "Freq" button offers the best performance at

the expense of a slight degradation of the signal amplitude accuracy and refresh rate.

The "Time" button selects the FFT algorithm of previous software versions (FFT with Hann window)

The "Mix" and "Freq" buttons select a polyphase FFT analysis with increasing window length (Polyphase FFT

with quasi flat-top amplitude response)

# OTHER FEATURES/NOTES

- Minor bugs corrected on perseususb.dll

- Perseus software does not hang anymore when a USB device (ie a USB memory stick) is disconnected

from the system

- Main window position stored on the registry. Desktop width checked and position

set to upper-left corner if it is not greater than 1024 pixels (minimum supported screen width).

- Status of AVG and NB buttons stored in the registry and reloaded at the application startup

- AGC not reset when mode/and or AGC constant changed to allow faster audio response.

- Occasional bad IQ data stream fed to the software from the Perseus hardware when reconfiguring its sample rate.

The bad IQ data stream can be observed like a "mirror like" spectrum on the main spectrum display.

When this happens, please select again the desired sample rate.

- Main Display Left/Right arrows always blocked when recording a file to prevent that frequency outside

the allowed range to be selected clicking these arrows.

_______________________________________________________________________________________________

# VIRTUAL CI-V INTERFACE - INTERFACE TO THIRD PARTY APPLICATIONS

_______________________________________________________________________________________________

Support for controlling the Perseus software (and the receiver itself) with third party software has been added via a virtual CI-V interface.

The Perseus software listen to the COM10 Virtual Com Port for third party commands and sets its operating frequency and mode accordingly to the received commands. This allow, i.e., to control the Perseus software via an external decoder application like MixW. To use such a feature a null modem emulator like com0com has to be installed into the system. Com0com is freely available under the GPL license here: http://sourceforge.net/projects/com0com/

Com0com installs two Virtual Communication ports CNCA0 and CNCB0: one of them is used from the Perseus software to listen to incoming commands, the second one is used by the external application to send commands to and get replies from the Perseus software.  Since Perseus listen always on the COM10 port, whether it is a physical or a virtual one, it is necessary to rename the default com0com ports to COM10 (used by Perseus) and to COM<xx> (the port that will be used by the third party software) where <xx> is a number which does not conflict with existing physical com ports on your system, i.e. COM11. To rename the two ports run the utility setupc.exe in the directory where com0com has been installed and digit the two commands:

change CNCA0 PortName=COM10

change CNCB0 PortName=COM11

Once the ports have been renamed, the third party software can control the Perseus application connecting (as in this example) to the COM11 com port.

If the Perseus software finds the COM10 port on the system it displays the label "VCom On" in the low-right corner of the main window. If this label shows "VCom Off", the Perseus software has not found the virtual COM10 port on the system. In this case the CI-V interface cannot be used.

The CI-V commands implemented in this software version are:

CMD-ID	COMMAND

$00		(Transfer operating frequency)

$01		(Transfer operating mode)

$03		(Read operating frequecny)

$04		(Read mode)

$05		(Set operating frequency)

$06		(Set operating mode)

$11		(Set/Get attenuator settings)

$15 $02 		(Read SMeter Level)

$16 $02 		(Set ADC Preamp and Dither Generator)

$16 $12 		(Set AGC)

$19 $00		(Read default address)

$70 $01 		(Set DDC Output Sample Rate) - Microtelecom extension

$70 $02 		(Start recording) - Microtelecom extension

$70 $03 		(Stop  recording) - Microtelecom extension

CI-V COMMAND PARAMETERS

-SET MODE COMMANDS  (CMD $01 and CMD $06)

Commands $01 and $06 ignore the filter passband parameter and only set the Perseus receiver

to the given mode accordingly to the following table:

MODE-ID 	MODE

$00		LSB

$01		USB

$02		AW

$03		CW

$04		RTTY

$05		FM

$06		SAM

$07		DRM

$08		USER (Zero IF, 31.25 KS/s, IQ output to VAC)

-SET ATTENUATOR COMMAND (CMD $11)

The set attenuator command recognizes the following data field:

DATA    ACTION

$00	Attenuator OFF

$10     	10 dB Attenuator

$20     	20 dB Attenuator

$30     	30 dB Attenuator

--     	Read attenuator settings

-READ S-METER LEVEL (CMD $15 $02)

When this command is issued Perseus reply with a 1 or 2 digits bcd value in the range 0-255

which is a scaled version of the received power.

0 corresponds to  -140 dBm

255 corresponds to +30 dBm

-SET ADC PREAMP AND DITHER GENERATOR (CMD $16 $02)

This command recognize the following data field

DATA	ACTION

$00	ADC Preamp OFF/Dither OFF

$01	ADC Preamp ON/Dither OFF

$02	ADC Preamp OFF/Dither ON

$03	ADC Preamp ON/Dither ON

-SET AGC (CMD $16 $12)

This command recognize the following data field

DATA	ACTION

$00	AGC OFF

$01	AGC fast

$02	AGC medium

$03	AGC slow

-READ DEFAULT ADDRESS COMMAND (CMD $19 $00)

The Perseus receiver replies to this command with the address $E1

-MICROTELECOM EXTENSIONS TO CI-V Protocol

-SET DDC OUTPUT SAMPLE RATE ( CMD $70 $01)

This command accepts the following data field

DATA		DDC OUT SAMPLE RATE

$00		125 kS/s

$01   		250 kS/s

$02   		500 kS/s

$03     		1 MS/s

-START RECORDING (CMD $70 $02)

The data field of this command must contain a null terminated file name to be created.

The Perseus software returns a NOK reply if the file cannot created (i.e. the path does not exist).

When a START RECORDING command has been issued some subsequent commands maybe not effective (i.e.

trying to change the frequency outside the recording bandwidth or to modify attenuator/adc settings).

Files are recorded in the same Perseus recorder style, with an added suffix _nnn prior to the .wav

file suffix, where nnn is the sequence number of the current record.

-STOP RECORDING (CMD $70 $02)

This command stops the recording.

NOTES:

- The Perseus software ignores the "To address" field of the CI-V protocol and responds to

any "To address" set in the received CI-V command.

- The current software version has been tested only with the com0com null modem emulator.

The current Perseus virtual CI-V interface does not work with Ham Radio DeLuxe. Don't ask me why.

Enjoy Perseus and 73 de Nico Palermo, IV3NWV – Microtelecom s.r.l. – Pavia di Udine, Italy